Road Curvature Model Optimization

Abstract

Background

In the United States, the prevailing standards for road design come from The American Association of State Highway and Transportation Officials, referred as the Green Book. This book offers an extensive review of road design considerations that comply with vehicle dynamic behavior. Special consideration is given to curve maneuvering because there are centripetal forces that need to be balanced with a combination of road factors to maintain vehicle stability. The dynamics are formulated using Newton’s Second Law of motion, which takes into consideration both road and vehicle characteristics. Road design parameters include, road friction, superelevation, and maximum width, while the vehicle parameters considered are velocity, acceleration, trackwidth, and vehicle length. These are summarized with the following formula []:

v2gρ=μ+0.01e1-0.01μe        Eq.1

Where:

v= Vehicle velocity (m/s)

e=Superelevation (as a percentage)

g= Gravitational acceleration (9.81 m/s2)

μ=Coefficient of side road friction

ρ= Radius of curvature (m)

This formula relates most of the parameters that can be involved from any general vehicle and general street. In the case of street design, road friction and superelevation are already implemented on most roads. Vehicles can vary their velocity and heading accordingly as they traverse any curve. However, the factor that unites both the road and the vehicle is the radius of curvature. From geometric considerations and Newton’s Second Law, it is possible to find another equation that relates more vehicle parameters to the radius of curvature as follows []:

δ= 57.3L+UGv2ρ-1       Eq.2

Where:

δ= Wheel directional angle (deg)

ρ= Radius of curvature (m)

v= Vehicle velocity (m/s)

L = Vehicle length (m)

UG= Understeer gradient (deg-s2/m)

Since the Radius of Curvature belongs to the set of all real numbers, to avoid division by zero, the inverse function is used. This is known as Curvature and is depicted as follows: κ=ρ-1 (m-1). Curvature is unique since every vehicle creates its own curvature when traversing any arbitrary road. However, all roads have already a pre-determined curvature that was designed for a variety of vehicles as discussed before. Furthermore, curvature can be directly related to a vehicle’s heading angle through an orthogonal shift for an instantaneous point in time. For this reason, being able to provide vehicles with this pre-determined curvature poses a new guiding factor that autonomous vehicle technology can implement into their decision algorithms for navigation. This in turn can improve their reliability under conditions where disruptions such as weather effects or poor lane markings disable sensor information.

There exist multiple types of roads in which curvature changes as a function of segment length (i.e. κ(s) ) to provide both stability and comfort to drivers. These are divided in two categories, horizontal and vertical curves. Horizontal curves focus on the direction of the centerline, while vertical curves focus on the slope of the centerline. Horizontal curves are divided into 4 main categories shown in Figure 1. For this project, only the 4 main horizontal curves are considered because they constitute most of the implemented roads available.

Figure 1. Horizontal Curves

Thus, curvature models were developed that can represent these types of roads. However, to attain an optimized curvature model that can be applied to general vehicles. An optimization problem needs to be defined that utilizes both real road data and vehicle parameters.

Optimization Problem Formulation

To obtain the road data, a previous study has been performed which outputs the curvature values needed to be compared with the curvature models []. These models are subject to an unconstrained Least Squares Error - Minimization problem such as:

minxκm(s)-κ2    Pr.1

Where:

κm(s)=  Road curvature model m

κ= Road sampled curvature data

This minimization problem (Pr.1) will test different analytical curvature models and obtain appropriate parameters for each of them accordingly. It is important to note that the models κm can be either linear or non-linear.

Pr.1 will focus on representing models κm as a representation of any generic road data input. However, to take into consideration the vehicle dynamic stability, Eq.1 and Eq.2 are used as a second minimization problem (Pr.2) that will minimize the steering wheel angle δ with the following constraints:

minx x1- 57.3L+UGx22κm(s)                Pr.2

subject to:

x22gκm(s)-μ+0.01e1-0.01μe=0

This will find the optimized combination for both traveling velocity and wheel angle that uses the model κm as part of their process. Pr. 2 is a non-linear constrained optimization problem in which the vehicle parameters (L,UG) and road parameters (μ,e) are regarded as constants for any generic road/vehicle. In general, there exists design vehicle ranges, and steering wheel angles such that extra constraints could be added to Pr. 2 in the following manner:

vmin<x2<vmax     &       δmin<x1<δmax        C.1

Combining Pr. 2 and C.1 it is possible to form an iterative method in which the velocity and wheel angle of a vehicle, can be optimized with respect to a previously optimized curvature profile.

Road Curvature Models

All horizontal curves (in Figure 1) are made of curvatures with constant, and/or linear slopes. To create a model to be optimized in Pr. 1, the following model is proposed:

Piece-wise linear model:

κ1=x5x2-x1s-x1φs-x1-φs-x2+x5φs-x2-φs-x3+x5x4-x3-s-x3+x5φs-x3-φs-x4    M.1

Where:

φx-a= Unit Step Function with a-shift

xi= Variables that describe curvature function with i∈[1,5]

M.1 was mathematically designed to be continuous for all s and offers the flexibility of having its xi parameters to be easily identified as basic geometric properties of a trapezoid. This is illustrated in Figure 3 for the general model κ1 along s with parameters xi.

The typical ranges for the variables are shown with a contour example in which Pr.2

Implementation

A contour plot of Pr. 2 is shown in Figure 2, with ranges that satisfy C.1. It is noted that the minimizer of Pr. 2 lies somewhere in the line generated from the intersection of Eq. 1 and Eq. 2.

Results

Conclusions

Code

MAX TEN PAGES 10

Mention different types of curves

Mention how aashto green book uses its own criterion to develop their design standards for friction

Usually these are made in a underestimated considerations (conservative values).

Mention how the values that we have try to find the optimal curvature and optimal travel velocity (because both velocity and heading angle are the only variables that we cant control during driving).

Some variables we cannot control:

Understeer gradient

Coefficient of Friction

Jacome, R., Stolle, C. and Sweigard, M., “Road Curvature Decomposition for Autonomous Guidance,” SAE Technical Paper

2020-01-1024, 2020, doi:10.4271/2020-01-1024.

Gillespie, T.D., Fundamentals of Vehicle Dynamics (SAE International, 1992). ISBN:1-56091-199-9.

A Policy on Geometric Design of Highways and Streets (The Green Book) Sixth Edition (American Association of State Highway and Transportation Officials, 2011).